PRACTICAL – 7

Interfacing serial port.

CODE:-

/*Baud Rate GENERATION

* n    => required baudrate

* BRGH     =   0

* SPBRG    =   (Fosc / (64 * n)) -1

* For 9600 baudrate, SPBRG ~=77

*/

#include<p18F4550.h>

#include<stdio.h>

#define Fosc 48000000UL

void InitUART(unsigned int baudrate)

{

TRISCbits.RC6 = 0;                        //TX pin set as output

TRISCbits.RC7 = 1;                        //RX pin set as input

SPBRG = (unsigned char)(((Fosc /64)/baudrate)-1);

BAUDCON = 0b00000000;                     //Non-inverted data; 8-bit baudrate generator

TXSTA = 0b00100000;                       //Asynchronous 8-bit; Transmit enabled; Low speed baudrate select

RCSTA = 0b10010000;                       //Serial port enabled; 8-bit data; single receive enabled

}

void SendChar(unsigned char data)

{

while(TXSTAbits.TRMT == 0);             //Wait while transmit register is empty

TXREG = data;                           //Transmit data

}

void putch(unsigned char data)

{

SendChar(data);

}

unsigned char GetChar(void)

{

while(!PIR1bits.RCIF);                  //Wait till receive buffer becomes full

return RCREG;                           //Returned received data

}

void main(void)

{

InitUART(9600);

printf("\r\nHello MicroPIC-18F: Enter any Key from Keyboard\r\n");

while(1)

{

printf("%c",GetChar());             //Receive character from PC and echo back

}

while(1);

}