PRACTICAL – 8

Interface analog voltage 0-5V to internal ADC and display value on LCD.

CODE:-

#include <p18f4550.h>

#include<stdio.h>

#define LCD_EN LATAbits.LA1

#define LCD_RS LATAbits.LA0

#define LCDPORT LATB

void lcd_delay(unsigned int time)

{

unsigned int i , j ;

for(i = 0; i < time; i++)

{

for(j=0;j<50;j++);

}

}

void SendInstruction(unsigned char command)

{

LCD_RS = 0;		// RS low : Instruction

LCDPORT = command;

LCD_EN = 1;		// EN High

lcd_delay(10);

LCD_EN = 0;		// EN Low; command sampled at EN falling edge

lcd_delay(10);

}

void SendData(unsigned char lcddata)

{

LCD_RS = 1;		// RS HIGH : DATA

LCDPORT = lcddata;

LCD_EN = 1;		// EN High

lcd_delay(10);

LCD_EN = 0;		// EN Low; data sampled at EN falling edge

lcd_delay(10);

}

void InitLCD(void)

{

TRISB = 0x00; //set data port as output

TRISAbits.RA0 = 0; //EN pin

TRISAbits.RA1 = 0; // RS pin

SendInstruction(0x38);      //8 bit mode, 2 line,5x7 dots

SendInstruction(0x06);	// entry mode

SendInstruction(0x0C);	//Display ON cursor OFF

SendInstruction(0x01);      //Clear display

SendInstruction(0x80);      //set address to 0

}

void ADCInit(void)

{

TRISEbits.RE1 = 1;                  //ADC channel 6 input

TRISEbits.RE2 = 1;                  //ADC channel 7 input

ADCON1 = 0b00000111;                //Ref voltages Vdd & Vss; AN0 - AN7 channels Analog

ADCON2 = 0b10101110;                //Right justified; Acquisition time 4T; Conversion clock Fosc/64

}

unsigned short Read_ADC(unsigned char Ch)

{

ADCON0 = 0b00000001 | (Ch<<2);      //ADC on; Select channel;

GODONE = 1;              //Start Conversion

while(GO_DONE == 1 );    //Wait till A/D conversion is complete

return ADRES;                       //Return ADC result

}

void DisplayResult(unsigned short ADCVal)

{

unsigned char i,text[16];

unsigned short tempv;

tempv = ADCVal;

SendInstruction(0x80);                 //set to 1st line

for(i=0;i<10;i++)                      //Display the 10 bit ADC result on LCD

{

if(tempv & 0x200)

{

SendData('1');

}

else

{

SendData('0');

}

tempv = tempv<<1;

}

ADCVal = (5500/1024)*ADCVal;                   //Convert binary data to mV;   1 bit <=> (5500/1024)mV

sprintf(text,"ADC value=%4dmv",ADCVal);         //Convert integer data to string

SendInstruction(0xC0);                     //set to 2nd line

for(i=0;i<16;i++)                          //Display string on LCD

{

SendData(text[i]);

}

}

void main()

{

unsigned short Ch_result;

TRISB = 0x00;                       //PORTB connected to LCD is output

ADCInit();

InitLCD();

while(1)

{

Ch_result = Read_ADC(7);

DisplayResult(Ch_result);

lcd_delay(1000);

}

}